OSCN LAB – 5

Name: SANIYA TAHSEEN                                   Date: 11/11/2025

HALL TICKET NO.: 2503B05122

PROGRAM 4 – To implement Hamming Code for Error Detection and Correction.

CODE:

// LAB 5 : PROGRAM 4

// To implement Hamming Code for Error Detection and Correction.

#include <iostream>     // For input/output operations

#include <cmath>        // For power function pow()

#include <vector>       // For using dynamic arrays (vector)

using namespace std;

// Function to calculate how many redundant bits are needed

int calcRedundantBits(int m) {

int r = 0;                       // Start with 0 redundant bits

// Keep increasing r until 2^r >= m + r + 1

while (pow(2, r) < (m + r + 1)) {

r++;                         // Increase redundant bit count

}

return r;                        // Return final redundant bits count

}

// Function to insert parity (redundant) bits into correct positions

vector<int> insertParityBits(const vector<int>& data, int r) {

int m = data.size();             // Number of original data bits

int total = m + r;               // Total bits including data + redundant

vector<int> hamming(total + 1);  // 1-indexed vector for convenience

int j = 0;                       // Pointer for data bits

// Place data bits and parity bit placeholders

for (int i = 1; i <= total; i++) {

// Positions that are powers of 2 (1,2,4,8...) are parity bits

if ((i & (i - 1)) == 0) {

hamming[i] = 0;          // Set parity bit placeholder

} else {

hamming[i] = data[j++];  // Fill data bit

}

}

return hamming;                  // Return packet with parity spots

}

// Function to calculate parity bits (EVEN parity)

void setParityBits(vector<int>& hamming, int r) {

int total = hamming.size() - 1;  // Total bits (1-indexed)

// For each parity bit

for (int i = 0; i < r; i++) {

int pos = pow(2, i);         // Position of parity bit (1,2,4,8...)

int count = 0;               // Count of 1s for parity check

// Check bits that influence this parity bit

for (int j = 1; j <= total; j++) {

if (j & pos) {           // If this bit participates in parity

count += hamming[j]; // Add the bit value

}

}

hamming[pos] = count % 2;    // Set even parity ? parity = count mod 2

}

}

// Function to detect error at receiver side

int detectError(const vector<int>& hamming, int r) {

int total = hamming.size() - 1;  // Total bits

int errorPos = 0;                // Store error position

// Recalculate parity bits

for (int i = 0; i < r; i++) {

int pos = pow(2, i);         // Parity position

int count = 0;               // Count of 1s

// Check all bits related to this parity bit

for (int j = 1; j <= total; j++) {

if (j & pos) {           // If bit influences this parity bit

count += hamming[j];

}

}

// If parity doesn't match (count should be even)

if (count % 2 != 0) {

errorPos += pos;         // Add position to error index

}

}

return errorPos;                 // Return final error location

}

int main() {

string input;

cout << "Enter binary data: ";   // Ask user for binary string

cin >> input;                    // Read the input

vector<int> data;                // Vector to store bits

// Convert string characters to integers (0 or 1)

for (char c : input) {

data.push_back(c - '0');

}

int m = data.size();             // Number of data bits

int r = calcRedundantBits(m);    // Calculate redundant bits

// Insert parity bits into correct positions

vector<int> hamming = insertParityBits(data, r);

// Set actual parity values

setParityBits(hamming, r);

// Display final packet to be sent

cout << "\nThe data packet to be sent is: ";

for (int i = 1; i < hamming.size(); i++) {

cout << hamming[i] << " ";

}

cout << "\n";

// Receiver-side simulation

cout << "\nSimulating receiver...\n";

int errorPos = detectError(hamming, r); // Recheck parity bits

// If errorPos is 0 ? no error found

if (errorPos == 0) {

cout << "Correct data packet received\n";

} else {

cout << "Error detected at position: " << errorPos << "\n";

cout << "Correcting error...\n"

// Flip the erroneous bit (0?1 or 1?0)

hamming[errorPos] ^= 1;

// Display corrected packet

cout << "Corrected data packet: ";

for (int i = 1; i < hamming.size(); i++) {

cout << hamming[i] << " ";

}

cout << "\n";

}

return 0;                         // End of program

}

//Time Complexity = O(nlogn)

//where , n = databits + redundant bits

OUTPUT:

____________________________________________________________________________________